# Experiment 	to test strength of gripper finger

!!Placeholder images of an older design!!

To test the strength of the fingers of 
the gripper, we will mount the bottom/gear in a fixture and then apply a force on the tip of the finger. Where the torque on the finger will be
at its largest.

We will be doing 4 test from different angles to evaluate the different cases of collision with the finger.

(1): Standard collision from closing on external diameter of a checkers piece.

(2): Collision when opening. I.e Gripping the internal diameter of a part.

(3): Collision when closing. I.e Gripping the external diameter that is larger than a checkers piece.

(4): Collision when moving down to collect piece.

There is also an argument to be made of collisions with a part in the gripper. But these are more complex and seen as out of the scope of the analysis.

4

3

2

1

Green: Force applied & measured with dynamic force cell
Red: Fixture positioning
Black: Reference lines